# Trial 3 Script 06 Work Process Report — Independent MS Grid Detection

This report documents the workflow used to detect the complete 7 × 10 multispectral tray grid for Trial 3. The method operates directly on the cropped MS_NIR image and does not transfer RGB/D coordinates.

# 1. Reason for replacing the earlier method

The earlier RGB-guided method produced a mathematically complete 70-cell grid but did not cover the true rightmost column and bottom row in some MS images. This happened because RGB and multispectral crops have different margins and camera geometry. A correct MS grid must therefore be detected independently within each MS image.

# 2. Input data

Cropped image folder: C:\Users\tshib\OneDrive\Desktop\Internship\outputs\Third trial\01_Crop_Dual_Reference

Each complete multispectral set contains MS_G, MS_R, MS_RE, and MS_NIR. MS_NIR is used as the grid-reference band because the tray cups and soil normally produce a repeated dark circular pattern against the lighter tray.

# 3. Automatic detection process

The MS_NIR image is normalised to 8-bit only for grid detection and overlay creation; original band values are not modified.

A darkness response map is created while reducing slow illumination variation.

Horizontal and vertical darkness profiles are calculated across the full image.

The script searches for the strongest regular ten-column and seven-row lattice, including the true outer rows and columns.

Each predicted cell centre is locally refined within a limited search radius so it cannot drift to a neighbouring cell or handwritten label.

A robust smooth polynomial surface is fitted to all 70 refined centres. Outliers are controlled through iterative residual filtering.

Polygon ownership zones are generated around every final centre for later NDVI and NDRE extraction.

Strict outer-coverage checks prevent a compressed grid from being accepted.

If the profile result is uncertain, Hough-circle configurations are evaluated separately. Their detections are never merged across configurations.

# 4. Validation status summary

| Status | Count |
| --- | --- |
| PASS_AUTO | 84 |

PASS_AUTO and PASS_MANUAL are acceptable inputs for Script 07 after visual overlay checking. CHECK_AUTO and FAIL rows should not be used for vegetation-index extraction until corrected.

# 5. Generated files

ms_grid_manifest.csv: One row per MS image set, including method, status, coverage, spacing, contrast, fit residuals, and overlay path.

ms_cell_coordinates.csv: One row per cell, containing the final MS-specific centre, polygon corners, bounding box, image paths, and validation status.

ms_cell_grid_report.xlsx: Excel workbook containing the manifest, coordinate table, status summary, and read-me guidance.

ms_independent_grid_work_process.docx: This work-process report describing the method and validation requirements.

MS_NIR grid overlay images: Visual evidence showing whether all seven rows and ten columns are correctly covered.

# 6. Visual checking requirements

Inspect overlays from different days and trays. Cell 1 must correspond to the real top-left cup, Cell 10 to the real top-right cup, Cell 61 to the real bottom-left cup, and Cell 70 to the real bottom-right cup. The handwritten label must not be interpreted as an additional column.

# 7. Use in Script 07

Script 07 should use the polygon columns in ms_cell_coordinates.csv. Observed multispectral values must be extracted from the original TIFF bands, not from the normalised overlay image.

# 8. Limitations

Strong image cropping errors, severe rotation, incomplete trays, or unusual lighting can still produce CHECK_AUTO or FAIL. Only those exceptional image sets should use targeted manual four-corner correction.